APB Memory Slave

Test Plan

Version 1.0  |  Project: APB VIP Practice

# 1. Priority Legend

| Priority | Meaning |
| --- | --- |
| P1 | Must pass — blocks tape-out |
| P2 | Should pass — important feature |
| P3 | Nice to have — corner case / coverage closure |

# 2. Basic Write / Read-Back Tests

Verify fundamental APB write and read transactions against the shadow memory scoreboard.

| Test Name | Purpose | Type | Priority | Pass Condition |
| --- | --- | --- | --- | --- |
| test_wr_rd_single | Write one word, read back, compare | Directed | P1 | PRDATA == PWDATA, 0 SB errors |
| test_wr_rd_random_addr | 20 random aligned addresses write then read | C-Random | P1 | All read-back values match shadow |
| test_wr_rd_all_zero | Write 0x00000000 to 16 addresses, read back | Directed | P1 | PRDATA == 0x00000000 |
| test_wr_rd_all_ones | Write 0xFFFFFFFF to 16 addresses, read back | Directed | P1 | PRDATA == 0xFFFFFFFF |
| test_wr_rd_alternating | Write 0xAAAA5555 / 0x5555AAAA alternating pattern | Directed | P2 | Data integrity, no bit flip |
| test_wr_rd_walking_bit | Walking-1 and walking-0 patterns across all data bits | Directed | P2 | Each bit toggled correctly |

# 3. PSTRB Byte-Enable Tests

Verify that PSTRB correctly enables individual byte lanes during writes.

| Test Name | Purpose | Type | Priority | Pass Condition |
| --- | --- | --- | --- | --- |
| test_pstrb_byte0_only | Write only byte 0 (PSTRB=0x1), read back | Directed | P1 | Only byte 0 updated in shadow |
| test_pstrb_byte3_only | Write only byte 3 (PSTRB=0x8), read back | Directed | P1 | Only byte 3 updated in shadow |
| test_pstrb_half_word_lo | Write lower 2 bytes (PSTRB=0x3) | Directed | P1 | Bytes 0-1 updated, 2-3 unchanged |
| test_pstrb_half_word_hi | Write upper 2 bytes (PSTRB=0xC) | Directed | P1 | Bytes 2-3 updated, 0-1 unchanged |
| test_pstrb_random | Random PSTRB for 30 transactions | C-Random | P2 | Byte-level scoreboard passes |
| test_pstrb_overlap | Write partial bytes then overwrite with different strobe | Directed | P2 | Merged result matches shadow |

# 4. PREADY / Wait State Tests

Verify master correctly waits for PREADY and that data is stable when PREADY asserts.

| Test Name | Purpose | Type | Priority | Pass Condition |
| --- | --- | --- | --- | --- |
| test_wait_0_cycles | WAIT_CYCLES=0: immediate PREADY | Directed | P1 | Transaction completes in 2 cycles |
| test_wait_1_cycle | WAIT_CYCLES=1: one wait state | Directed | P1 | PREADY held low for 1 extra cycle |
| test_wait_3_cycles | WAIT_CYCLES=3 (default config) | Directed | P1 | PREADY low for 3 cycles, data correct |
| test_wait_random | Random WAIT_CYCLES 0-7, 20 transactions | C-Random | P2 | No timeout, data always correct |
| test_prdata_stable | Confirm PRDATA stable for entire PREADY=1 cycle | Directed | P2 | No glitch on PRDATA during valid window |

# 5. Protocol Compliance Tests

Verify the DUT and master both comply with the APB4 protocol state machine.

| Test Name | Purpose | Type | Priority | Pass Condition |
| --- | --- | --- | --- | --- |
| test_penable_timing | PENABLE asserts exactly one cycle after PSEL | Directed | P1 | SVA p_penable_after_psel passes |
| test_addr_stable_access | PADDR/PWRITE stable throughout access phase | Directed | P1 | SVA p_stable_in_access passes |
| test_back_to_back | 10 consecutive transactions with no IDLE gap | Directed | P2 | All transactions captured by monitor |
| test_idle_between | Random IDLE cycles (1-5) between transactions | C-Random | P2 | No data corruption across transactions |
| test_read_before_write | Read from uninitialised address, expect 0 | Directed | P2 | PRDATA == 0x00000000, no SB error |

# 6. Reset Tests

Verify correct behaviour when PRESETn is asserted during normal operation.

| Test Name | Purpose | Type | Priority | Pass Condition |
| --- | --- | --- | --- | --- |
| test_reset_at_startup | Assert reset before first transaction | Directed | P1 | wait_cnt=0, no spurious PREADY |
| test_reset_mid_setup | Assert reset during SETUP phase | Directed | P1 | Transaction aborted, bus returns to IDLE |
| test_reset_mid_access | Assert reset during ACCESS phase (PREADY=0) | Directed | P1 | Transaction aborted cleanly |
| test_reset_then_resume | Reset, then run 10 normal write/read transactions | Directed | P2 | All post-reset transactions pass SB |

# 7. Address Boundary Tests

Verify correct behaviour at the extremes of the supported address space.

| Test Name | Purpose | Type | Priority | Pass Condition |
| --- | --- | --- | --- | --- |
| test_addr_min | Write/read at address 0x000 | Directed | P1 | Data correct |
| test_addr_max | Write/read at address 0xFFC (top of 1K space) | Directed | P1 | Data correct |
| test_addr_full_sweep | Sequential write then read across all 1024 words | Directed | P2 | 0 SB errors across entire memory |
| test_addr_random_dense | 100 random writes then matching reads | C-Random | P3 | Coverage closure on address bins |

# 8. PSLVERR Tests

Verify PSLVERR is never incorrectly asserted (DUT hardwires it to 0).

| Test Name | Purpose | Type | Priority | Pass Condition |
| --- | --- | --- | --- | --- |
| test_pslverr_never_set | Run 50 random transactions, check PSLVERR=0 always | C-Random | P1 | SVA p_pslverr_only_access passes; SB 0 errors |
| test_pslverr_on_read | Confirm PSLVERR=0 on all read transactions | Directed | P2 | No unexpected PSLVERR captured |
| test_pslverr_on_write | Confirm PSLVERR=0 on all write transactions | Directed | P2 | No unexpected PSLVERR captured |

# 9. Regression Summary

| Category | Total Tests | P1 | Automation |
| --- | --- | --- | --- |
| Basic Write/Read | 6 | 4 | UVM regression |
| PSTRB Byte-Enable | 6 | 4 | UVM regression |
| PREADY / Wait State | 5 | 3 | UVM regression |
| Protocol Compliance | 5 | 3 | UVM regression + SVA |
| Reset | 4 | 2 | UVM regression |
| Address Boundary | 4 | 2 | UVM regression |
| PSLVERR | 3 | 1 | UVM regression + SVA |
| TOTAL | 33 | 19 | xrun -f apb.f +UVM_TESTNAME=... |