APB Memory Slave

Verification Specification

Version 1.0  |  Project: APB VIP Practice

| Field | Value |
| --- | --- |
| Document Owner | DV Engineer |
| DUT | apb_memory_slave (APB4 Slave, 32-bit data, 10-bit address) |
| Protocol | AMBA APB4 (PREADY / PSLVERR / PSTRB support) |
| Simulator | Cadence Xcelium 23.03 + UVM 1.1d |
| Created | 2026-05 |

# 1. Overview

This document defines the verification strategy for the APB memory slave DUT (apb_memory_slave). The DUT implements an AMBA APB4-compliant slave interface with configurable wait states (WAIT_CYCLES), byte-enable write support (PSTRB), and PSLVERR error signalling. The verification environment is built with SystemVerilog UVM 1.1d.

# 2. DUT Architecture

## 2.1 Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| ADDR_WIDTH | 10 | Address bits; memory depth = 2^ADDR_WIDTH words |
| DATA_WIDTH | 32 | Data bus width in bits |
| WAIT_CYCLES | 0 | Number of wait states inserted before PREADY is asserted |

## 2.2 Interface Signals

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| PCLK | 1 | IN | Clock |
| PRESETn | 1 | IN | Active-low synchronous reset |
| PADDR | 32 | IN | Address bus |
| PSEL | 1 | IN | Slave select |
| PENABLE | 1 | IN | Enable / access phase indicator |
| PWRITE | 1 | IN | 1 = Write, 0 = Read |
| PWDATA | 32 | IN | Write data bus |
| PSTRB | 4 | IN | Byte-enable strobes (APB4) |
| PRDATA | 32 | OUT | Read data bus; combinational; valid when PREADY=1 |
| PREADY | 1 | OUT | Slave ready; combinational from wait_cnt |
| PSLVERR | 1 | OUT | Slave error; tied low in this DUT |

# 3. Verification Environment Architecture

## 3.1 Component Hierarchy

| Component | Role |
| --- | --- |
| top_tb | Top-level module; instantiates DUT and apb_if; starts UVM |
| apb_if | SystemVerilog interface; master_cb / monitor_cb clocking blocks; SVA assertions |
| apb_test | Test class; creates env; starts sequences |
| apb_env | Environment; instantiates agent and scoreboard |
| apb_agent (active) | Contains sequencer, driver, monitor |
| apb_driver | Drives APB master signals via master_cb clocking block |
| apb_monitor | Passive observation of all APB signals; publishes apb_item via analysis port |
| apb_scoreboard | Shadow memory comparison; checks PRDATA == expected on every read |
| apb_sequence | Generates write / read-back transaction pairs |
| apb_item | UVM sequence item: paddr, pwrite, pwdata, pstrb, prdata, pslverr |

## 3.2 Clocking Block Strategy

master_cb (output): drives PADDR, PSEL, PENABLE, PWRITE, PWDATA, PSTRB.

monitor_cb (input):  samples PADDR, PSEL, PENABLE, PWRITE, PWDATA, PSTRB, PREADY, PSLVERR with default input #1step.

PRDATA is sampled directly from the interface wire (vif.PRDATA) after wait(PREADY) to avoid clocking-block delta-cycle latency.

# 4. Coverage Strategy

## 4.1 Functional Coverage

| Cover Group | Cover Points |
| --- | --- |
| cg_transaction | pwrite (RD/WR), pstrb (0x0 / partial / 0xF), pslverr |
| cg_address | paddr: min (0x000), max (0xFFC), aligned, unaligned boundary |
| cg_wait_state | wait_cycles: 0, 1, 2, 3+ before PREADY assertion |
| cg_cross | pwrite X pstrb, pwrite X address range |
| cg_protocol | SETUP → ACCESS transition count, back-to-back transactions |

## 4.2 Code Coverage Target

- Line coverage:      100%

- Branch coverage:    100%

- Toggle coverage:    95%+ (clock / reset pins excluded)

- FSM coverage:       100% state and transition (IDLE / SETUP / ACCESS)

# 5. Assertion Strategy (SVA)

| Assertion | Description |
| --- | --- |
| p_penable_after_psel | PENABLE may only assert one cycle after PSEL rises |
| p_stable_in_access | PADDR, PWRITE, PWDATA, PSTRB must remain stable while PENABLE=1 and PREADY=0 |
| p_prdata_valid | PRDATA must not be X/Z when PSEL && PENABLE && !PWRITE && PREADY |
| p_pslverr_only_access | PSLVERR must be 0 outside the access phase |
| p_reset_idle | All master outputs must be 0 within one cycle of PRESETn deassertion |

# 6. Verification Exit Criteria

- All test cases in the Test Plan pass with 0 UVM_ERROR / UVM_FATAL

- Functional coverage closure: all covergroups ≥ 95%

- Code coverage: line/branch 100%, toggle 95%+

- All SVA assertions pass across entire regression

- Scoreboard reports 0 FAIL for all write-read-back tests

# 7. Risk Assessment

| Risk | Severity | Mitigation |
| --- | --- | --- |
| PRDATA timing (comb logic + clocking block) | High | Read vif.PRDATA wire directly after wait(PREADY) |
| WAIT_CYCLES off-by-one | Medium | Directed test with WAIT_CYCLES=0,1,2,3 and waveform check |
| PSTRB partial write corruption | Medium | Exhaustive PSTRB patterns with byte-level scoreboard |
| Back-to-back transaction address aliasing | Low | Random constrained address with overlap sequences |