AXI Verification Specification

& Test Plan

AMBA AXI4 / AXI4-Lite Protocol Compliance

| Document ID | AXI-VSPEC-001 |
| --- | --- |
| Version | 1.0 |
| Date | 2026-05-12 |
| Status | Draft |
| Author | Verification Team |

# 1. Introduction

## 1.1 Purpose

This document defines the verification strategy, test plan, and coverage goals for an IP block implementing the AMBA AXI4 / AXI4-Lite interface. It serves as the authoritative reference for the verification team throughout the project lifecycle.

## 1.2 Scope

- AXI4 Full (burst, outstanding transactions, out-of-order responses)

- AXI4-Lite (simplified, single-beat, no burst)

- Protocol compliance per ARM IHI0022H

- Functional coverage closure and regression sign-off

## 1.3 References

| Ref | Document | Version |
| --- | --- | --- |
| [1] | AMBA AXI and ACE Protocol Specification | IHI0022H |
| [2] | UVM User Guide | IEEE 1800.2-2020 |
| [3] | Project RTL Specification | v2.1 |

# 2. DUT Overview

## 2.1 Block Description

The Device Under Test (DUT) is an AXI4 interconnect/master/slave IP. It exposes one or more AXI4 manager (master) ports and one or more AXI4 subordinate (slave) ports.

## 2.2 Interface Summary

| Port Name | Direction | Protocol | Description |
| --- | --- | --- | --- |
| m_axi | Manager | AXI4 | Primary AXI4 manager port |
| s_axi | Subordinate | AXI4 | Primary AXI4 subordinate port |
| s_axi_lite | Subordinate | AXI4-Lite | Control/Status register port |
| clk | Input | — | System clock |
| aresetn | Input | — | Active-low synchronous reset |

# 3. Verification Architecture

## 3.1 Testbench Structure

The testbench is built on UVM (Universal Verification Methodology) and follows a layered architecture:

- Test Layer — top-level test classes, stimulus configuration

- Environment Layer — env, scoreboards, coverage collectors

- Agent Layer — AXI4 manager agent, AXI4 subordinate agent

- Sequence Layer — virtual sequences, scenario libraries

- Driver / Monitor / Sequencer — per-channel BFMs

## 3.2 Key Components

| Component | Responsibility |
| --- | --- |
| axi_master_agent | Drive AW/W/AR channels; collect B/R responses |
| axi_slave_agent | Respond to AW/W/AR; drive B/R channels with configurable latency |
| axi_scoreboard | Check data integrity; verify read-after-write consistency |
| axi_protocol_checker | Inline assertion-based protocol rule checker |
| axi_coverage | Functional coverage: burst types, sizes, responses, ordering |
| ref_model | Golden reference model for expected read data |

# 4. AXI Protocol Rules (Key Assertions)

The following protocol invariants must be verified via SVA (SystemVerilog Assertions) and/or checker logic:

## 4.1 Handshake Rules

| ID | Rule | Assertion | Priority |
| --- | --- | --- | --- |
| AX-001 | VALID must not deassert without READY (once asserted) | p_valid_stable: VALID && !READY ¦=> VALID | Critical |
| AX-002 | Data must not change while VALID && !READY | p_data_stable: VALID && !READY ¦=> $stable(data) | Critical |
| AX-003 | No X/Z on VALID/READY signals | p_no_x: !$isunknown(VALID) && !$isunknown(READY) | Critical |
| AX-004 | WLAST must match AWLEN beat count | p_wlast_count | High |
| AX-005 | RLAST must be asserted on final beat | p_rlast_count | High |
| AX-006 | BID/RID must match AWID/ARID | p_id_match | High |

## 4.2 Ordering Rules

- Write responses must be in order per AWID (same ID ordering).

- Read data must be in order per ARID (same ID ordering).

- Different IDs may be returned out of order (OOO support required for AXI4).

- No deadlock: the manager must not stall waiting for a response it hasn't issued.

# 5. Test Plan

## 5.1 Test Categories

| TC-ID | Test Name | Description | Type | Priority |
| --- | --- | --- | --- | --- |
| TC-001 | axi_single_write | Single AXI write transaction, fixed address, 1 beat | Smoke | P0 |
| TC-002 | axi_single_read | Single AXI read transaction, fixed address, 1 beat | Smoke | P0 |
| TC-003 | axi_write_read_back | Write then read same address; scoreboard checks data | Functional | P0 |
| TC-004 | axi_incr_burst_wr | INCR burst write: AWLEN = 1..255, all AWSIZE | Functional | P0 |
| TC-005 | axi_incr_burst_rd | INCR burst read: ARLEN = 1..255, all ARSIZE | Functional | P0 |
| TC-006 | axi_wrap_burst | WRAP burst: AWLEN = 1,3,7,15; address wrapping check | Functional | P0 |
| TC-007 | axi_fixed_burst | FIXED burst: all beats target same address | Functional | P1 |
| TC-008 | axi_narrow_transfer | AWSIZE < bus width; byte enable check | Functional | P1 |
| TC-009 | axi_unaligned_addr | Start address not aligned to transfer size | Functional | P1 |
| TC-010 | axi_outstanding_wr | Max outstanding write transactions simultaneously | Stress | P1 |
| TC-011 | axi_outstanding_rd | Max outstanding read transactions simultaneously | Stress | P1 |
| TC-012 | axi_ooo_response | Out-of-order read response; verify RID ordering | Functional | P1 |
| TC-013 | axi_interleaved_wr | Interleaved write data from multiple managers | Stress | P2 |
| TC-014 | axi_exclusive_access | AxLOCK = exclusive; verify EXOKAY / OKAY | Functional | P1 |
| TC-015 | axi_slave_backpressure | Slave holds READY low; verify VALID stability | Protocol | P0 |
| TC-016 | axi_error_response | RRESP/BRESP = SLVERR / DECERR; DUT handles gracefully | Error | P1 |
| TC-017 | axi_reset_sequence | Assert aresetn mid-transaction; verify clean recovery | Reset | P0 |
| TC-018 | axi_lite_rw | AXI4-Lite single write / read to CSR space | Smoke | P0 |
| TC-019 | axi_lite_simultaneous | Simultaneous LITE and full AXI4 transactions | Functional | P2 |
| TC-020 | axi_random_stress | Fully random stimulus, 10 K transactions | Random | P1 |

# 6. Functional Coverage Plan

## 6.1 Coverage Groups

| Covergroup | Coverpoints / Crosses | Target (%) |
| --- | --- | --- |
| cg_aw_channel | AWLEN(0,1,3,7,15,255), AWSIZE(0-6), AWBURST(FIXED/INCR/WRAP), AWLOCK, AWCACHE, AWPROT | 100% |
| cg_w_channel | WSTRB all combinations, WLAST, beat count vs AWLEN | 95% |
| cg_b_channel | BRESP (OKAY/EXOKAY/SLVERR/DECERR) x BID | 100% |
| cg_ar_channel | ARLEN, ARSIZE, ARBURST, ARLOCK, ARCACHE, ARPROT | 100% |
| cg_r_channel | RRESP x RLAST, RID interleaving | 100% |
| cg_outstanding | Num outstanding WR, Num outstanding RD, max depth | 95% |
| cg_cross_wr_rd | Simultaneous outstanding WR x RD (cross) | 90% |
| cg_latency | AW-to-W latency, AR-to-R latency buckets | 90% |
| cg_reset | Reset during: idle, AW phase, W phase, B phase, AR phase, R phase | 100% |

## 6.2 Code Coverage Goals

| Metric | Target | Notes |
| --- | --- | --- |
| Line Coverage | 100% | All RTL lines exercised |
| Toggle Coverage | ≥ 95% | Tied/power pins excluded |
| Condition Coverage | ≥ 90% | May waive unreachable conditions |
| FSM State Coverage | 100% | All legal FSM states |
| FSM Transition Coverage | ≥ 95% | Waive illegal/unused transitions |
| Branch Coverage | ≥ 95% |  |

# 7. Verification Sign-Off Criteria

## 7.1 Completion Checklist

- All P0 test cases pass with zero failures in regression.

- All P1 test cases pass with zero failures in regression.

- P2 test cases pass or have documented waivers with root-cause analysis.

- Functional coverage targets met per Section 6.

- Code coverage targets met; all waivers reviewed and approved.

- All SVA assertions enabled with zero vacuous pass rate > 0.1%.

- Protocol checker reports zero violations over all regression runs.

- Random seed sweep: ≥ 1000 unique seeds run with no new failures.

- Formal property verification (FPV) sign-off on channel handshake properties.

- All known bugs closed or deferred with written justification.

## 7.2 Regression Configuration

| Parameter | Value |
| --- | --- |
| Simulator | VCS / Xcelium / Questa (choose per site license) |
| UVM Version | IEEE 1800.2-2020 |
| Random Seeds | 1000 minimum for nightly; fixed seeds for CI |
| Timeout (per test) | 10 ms simulation time or 24 h wall-clock |
| Regression Trigger | On every RTL commit via CI/CD |
| Coverage Merge | Unified database across all seeds |

# 8. Verification Schedule

| Phase | Milestone | Duration | Owner |
| --- | --- | --- | --- |
| Phase 1 | TB architecture, VIP integration, smoke tests pass | 2 weeks | VE Lead |
| Phase 2 | P0 test suite complete, protocol assertions enabled | 3 weeks | VE Team |
| Phase 3 | P1/P2 tests, coverage closure, random sweep | 4 weeks | VE Team |
| Phase 4 | FPV, regression hardening, sign-off review | 2 weeks | VE Lead + Design |

# 9. Glossary

| Term | Definition |
| --- | --- |
| AXI | Advanced eXtensible Interface — AMBA bus protocol by ARM |
| AWID/ARID/WID/BID/RID | Transaction ID tags used for out-of-order support |
| BFM | Bus Functional Model — RTL-accurate driver/monitor |
| CSR | Control and Status Register |
| DUT | Device Under Test |
| FPV | Formal Property Verification |
| OOO | Out-Of-Order — responses returned in a different order than issued |
| SVA | SystemVerilog Assertion |
| UVM | Universal Verification Methodology |
| VIP | Verification IP — pre-verified, reusable TB component |

— End of Document —